Virtual Mini-Foundry Project Code Framework Guide

Based on the document "Virtual Mini-Foundry Code Overview", the following presents the complete code framework diagram and module function annotations. The diagram is divided into four levels: top-level directory structure, core data flow, 01_lithography lithography module, and 02_process_stages process chain module. In the diagram, "→" represents data flow direction, and parentheses indicate the core function of each module.

I. Top-Level Directory Structure

virtual_minifab_7mask_fullpackage/

│

├── 01_lithography/                     # Independent lithography teaching package (imaging, RET, process window)

│   ├── photolithography_baseline.py    # Core lithography simulator (partially coherent imaging, PEB, development)

│   ├── run_project.py                  # Lithography sub-project entry point, sweeps dose/defocus and outputs metrics

│   ├── mask_import.py                  # PNG mask → numpy array

│   ├── illumination.py                 # Illumination source sampling (annular, dipole)

│   ├── opc.py                          # Optical proximity correction (mask bias, morphological OPC)

│   ├── psm.py                          # Phase shift mask (1D/2D)

│   ├── smo.py                          # Source-mask co-optimization (grid search)

│   ├── spin_coating.py                 # Spin coating thickness model

│   ├── metrics.py                      # Process window area, NILS, CDU, and other evaluation metrics

│   └── tools/klayout_png_to_npy.py     # KLayout PNG to npy command-line tool

│

├── 02_process_stages/                  # Complete 7-mask process chain (etch → CVD → implant → metallization → device)

│   ├── run_7mask_pipeline.py           # Main scheduler, connecting all stages

│   ├── stage2_etch.py                  # Etch (anisotropy, loading effect, over-etch)

│   ├── stage3_cvd.py                   # CVD dielectric filling (conformality, HDP, void risk)

│   ├── stage4_implant_thermal_multimask.py   # Implant + anneal (ACTIVE/NIMP/PIMP mask combination)

│   ├── stage4_thermal_implant.py       # Detailed implant + anneal model (1D doping distribution, junction depth)

│   ├── stage5_metallization_cmp_multimask.py # Metallization + CMP (based on CONT/M1/V1 masks)

│   ├── stage5_metallization_cmp.py     # Detailed metallization + CMP model (dishing/erosion)

│   ├── stage6_device.py                # Device metric proxy model (Vt, Ioff, Ron, interconnect resistance)

│   ├── common_utils.py                 # Common utilities (JSON/NPY read/write, morphological operations)

│   ├── mask_ops.py                     # Mask geometry operations (bias, overlay, dilation/erosion)

│   └── plasma_basics_utils.py          # Plasma helper functions (mean free path, ion energy)

│

├── gds_router/                         # Layout → mask conversion

│   └── gds_to_masks.py                 # Read GDS and rasterize each layer into binary masks (.npy/.png)

│

├── configs/                            # Configuration file center

│   ├── process_flow.json               # 7-mask main process recipe (stage parameters, mask strategies)

│   ├── layer_map.json                  # Logical layer names → GDS layer/datatype mapping

│   ├── litho_config.json               # Lithography-specific parameters (wavelength, NA, dose range, etc.)

│   └── *example.json                   # Example templates (for students to copy and modify)

│

├── masks/                              # Stores generated mask files (.npy/.png)

├── gds/                                # Stores input GDS layouts (e.g., standard_cell.gds)

├── outputs/                            # Output directory for results

├── INSTALL.md                          # Environment setup and quick start guide

└── requirements.txt                    # Python dependency list

II. Core Data Flow (Main Flow)

┌─────────────────┐

│  GDS layout file │

│ (standard_cell.gds) │

└────────┬────────┘

         ↓

┌─────────────────────────────────────────┐

│ gds_router/gds_to_masks.py              │

│ Function: read GDS, use layer_map.json  │

│ to rasterize polygons of each layer     │

│ into binary .npy masks                  │

│ Output: masks/ACTIVE.npy, POLY.npy, ... │

└────────┬────────────────────────────────┘

         ↓

┌─────────────────────────────────────────┐

│ 02_process_stages/run_7mask_pipeline.py │

│ Function: main scheduler, sequentially  │

│ calls:                                 │

│   1. POLY lithography (calls 01_lithography) │

│   2. Implant + anneal (stage4)          │

│   3. Metallization + CMP (stage5)       │

│   4. Device metrics (stage6)            │

└────────┬────────────────────────────────┘

         ↓

┌─────────────────────────────────────────┐

│ 01_lithography/run_project.py           │

│ Function: full lithography simulation   │

│ for POLY layer                         │

│ Input: POLY mask + litho_config.json    │

│ Output: CD, process window, Leff, etc.  │

│ (poly_summary.json)                     │

└────────┬────────────────────────────────┘

         ↓

┌─────────────────────────────────────────┐

│ 02_process_stages/                      │

│ stage4_implant_thermal_multimask.py     │

│ Function: based on ACTIVE/NIMP/PIMP     │

│ masks, calculate doping window,         │

│ junction depth, sheet resistance        │

│ Output: device_inputs.json              │

└────────┬────────────────────────────────┘

         ↓

┌─────────────────────────────────────────┐

│ 02_process_stages/                      │

│ stage5_metallization_cmp_multimask.py   │

│ Function: based on CONT/M1/V1 masks,    │

│ calculate pattern density, post-CMP     │

│ copper thickness, interconnect          │

│ resistance proxy                        │

│ Output: metrics_stage5.json             │

└────────┬────────────────────────────────┘

         ↓

┌─────────────────────────────────────────┐

│ 02_process_stages/stage6_device.py      │

│ Function: map process results to        │

│ device metrics                          │

│ Output: metrics_stage6.json             │

│ (Vt, Ioff, Ron, etc.)                  │

└─────────────────────────────────────────┘

Note: The actual process flow also includes etch (stage2) and CVD (stage3), but for simplification the main flow directly proceeds to implantation. The complete sequence is: POLY lithography → etch → CVD → implant + anneal → metallization + CMP → device.

III. 01_lithography Module Function Annotations

File

Core Function

Input

Output

photolithography_baseline.py

Scalar imaging + simplified resist model

Mask (1D/2D), optical parameters, dose/defocus

Aerial image, post-development pattern, CD

run_project.py

Lithography sub-project entry

config.json (dose/defocus sweep range)

Process window images, metrics.json

mask_import.py

PNG mask import and preprocessing

PNG exported from KLayout

numpy binary mask

illumination.py

Illumination source sampling (annular/dipole)

sigma_in/out, n_r, n_theta

Normalized pupil coordinate points

opc.py

Optical proximity correction

Mask, target CD

Corrected mask (bias/morphology)

psm.py

Phase shift mask

Amplitude mask, phase regions

Complex transmission mask

smo.py

Source-mask co-optimization

Evaluation function, sigma_out list, bias list

Optimal parameters, heatmap

spin_coating.py

Spin coating thickness model

Viscosity, solid content, rotation speed, time

Thickness (nm), radial/2D distribution

metrics.py

Process window area, NILS, CDU, etc.

Pass/fail maps, CD arrays

Numerical metrics

IV. 02_process_stages Module Function Annotations

File

Core Function

Input

Output

run_7mask_pipeline.py

Main scheduler connecting all stages

process_flow.json

All stage outputs aggregated, pipeline_outputs.json

stage2_etch.py

Etch proxy model

Post-lithography resist pattern, pressure, power, magnetic field

Etched pattern, anisotropy index, over-etch selectivity requirements

stage3_cvd.py

CVD dielectric filling proxy model

Post-etch openings, conformality, temperature, HDP cycles

Dielectric thickness map, void risk map

stage4_implant_thermal_multimask.py

Implant + anneal (multi-mask version)

ACTIVE/NIMP/PIMP masks, poly_summary.json

Doping window, junction depth, activation rate, device_inputs.json

stage4_thermal_implant.py

Implant + anneal (1D detailed model)

Dose, energy, annealing temperature/time

Doping profile, junction depth, sheet resistance

stage5_metallization_cmp_multimask.py

Metallization + CMP (multi-mask version)

CONT/M1/V1 masks, overburden parameters

Pattern density, post-CMP copper thickness, interconnect resistance proxy

stage5_metallization_cmp.py

Metallization + CMP (detailed model)

Depth map, feature masks, overburden thickness

Copper thickness, post-CMP topography, dishing/erosion maps

stage6_device.py

Device metric proxy model

JSON outputs from process stages (Leff, xj, Rs, etc.)

Vt, Ioff, Ron, interconnect resistance

common_utils.py

Common utilities

JSON/NPY read/write, binarization, dilation/erosion, local feature size

File saving, image processing results

mask_ops.py

Mask geometry operations

Mask arrays, bias values, overlay shifts

Modified masks

plasma_basics_utils.py

Plasma helper functions

Pressure, power, magnetic field, temperature

Mean free path, ion energy, ion flux

The above framework diagram comprehensively reflects the module division, data flow, and core responsibilities of each file in the project. Students can use this to quickly locate the code areas they need to focus on and understand the chaining relationship of the process flow.